# Cisco Packet Tracer – Basic Networking Labs

# Lab 1: Simple LAN Setup

🎯 Goal: Connect two PCs through a switch and verify communication using ping.

🛠 Steps:

- Open Cisco Packet Tracer.

- Drag 2 PCs and 1 Switch.

- Use Copper Straight-Through cables to connect each PC to the switch.

- Assign IP addresses:
  - PC1: 192.168.1.1, Subnet: 255.255.255.0
  - PC2: 192.168.1.2, Subnet: 255.255.255.0

- From PC1, open Command Prompt and run: ping 192.168.1.2

- Expected: Ping should be successful.

- Output:

# Lab 2: Basic Router Setup (Connect Two LANs)

🎯 Goal: Connect two LANs using a router to simulate inter-network communication.

🛠 Steps:

- Use 2 PCs, 2 Switches, and 1 Router.

- Connect:
  - PC1 → Switch1 → Router (G0/0)
  - PC2 → Switch2 → Router (G0/1)

- Configure IPs:
  - PC1: 192.168.1.10, Gateway: 192.168.1.1
  - PC2: 192.168.2.10, Gateway: 192.168.2.1

- Router Configuration (CLI):

-   interface g0/0
  ip address 192.168.1.1 255.255.255.0
  no shutdown

-   interface g0/1
  ip address 192.168.2.1 255.255.255.0
  no shutdown

- From PC1, ping 192.168.2.10
Expected: Ping should succeed.

- Output:

# Lab 3: DNS Request Simulation

🎯 Goal: Simulate DNS resolution using a server and PC.

🛠 Steps:

- Use 1 PC, 1 Server, and 1 Switch.

- Assign IPs:
  - Server: 192.168.3.1
  - PC: 192.168.3.10, DNS: 192.168.3.1

- On Server > Services > DNS:
  - Add: Name = google.com, IP = 8.8.8.8

- On PC > Command Prompt, run: nslookup google.com

- Expected: Should return IP address 8.8.8.8.

- Output:

# Lab 4: DHCP Server Lab

🎯 Goal: Automatically assign IP to a PC using DHCP.

🛠 Steps:

- Use 1 PC, 1 Server, and 1 Switch.

- Server > Services > DHCP:
  Pool Name: Office
  Default Gateway: 192.168.5.1
  Subnet: 255.255.255.0
  Starting IP: 192.168.5.10

- Set PC to DHCP mode from IP Configuration.

- Expected: PC receives IP automatically (e.g., 192.168.5.10).

- Output:

# Lab 5: NAT Configuration

🎯 Goal: Use NAT to let private IP devices access a public server.

🛠 Steps:

- Use 1 Router, 1 PC (private), 1 Server (public).

- IP Plan:
  - PC: 192.168.10.10, Gateway: 192.168.10.1
  - Router G0/0: 192.168.10.1 (inside)
  - Router G0/1: 203.0.113.1 (outside)
  - Server: 203.0.113.5

- Router Configuration (CLI):

-   interface g0/0
  ip address 192.168.10.1 255.255.255.0
  ip nat inside
  no shutdown

-   interface g0/1
  ip address 203.0.113.1 255.255.255.0
  ip nat outside
  no shutdown

-   access-list 1 permit 192.168.10.0 0.0.0.255

-   ip nat inside source list 1 interface g0/1 overload

- From PC, ping 203.0.113.5
Expected: Ping should succeed via NAT.

- Output:

- 🧪 Lab 1: Simple LAN Setup

- 🎯 Objective

- To build a Local Area Network (LAN) by connecting two PCs through a switch and verify their connectivity using the ping command.

- 🛠️ Devices Used

| - Device Type | - Model | - Quantity |
| --- | --- | --- |
| - End Device | - PC (PC-PT) | - 2 |
| - Switch | - 2960 | - 1 |
| - Cable | - Copper Straight-Through | - 2 |

- 🔗 Network Topology

- PC0  ----

- |__Switch0__

- PC1  ----

- ⚙️ IP Address Configuration

| - Device | - IP Address | - Subnet Mask |
| --- | --- | --- |
| - PC0 | - 192.168.1.1 | - 255.255.255.0 |
| - PC1 | - 192.168.1.2 | - 255.255.255.0 |

- No gateway is required since both PCs are in the same subnet.

- 🧭 Step-by-Step Procedure

- Open Cisco Packet Tracer

- Launch the application.

- Add Devices

- Drag 2 PCs and 1 Switch into the workspace.

- Connect Devices

- Use Copper Straight-Through cables to connect:

- PC0 → Switch (FastEthernet0/1)

- PC1 → Switch (FastEthernet0/2)

- Assign IP Addresses

- Click on each PC → Desktop → IP Configuration:

- PC0: IP 192.168.1.1, Subnet 255.255.255.0

- PC1: IP 192.168.1.2, Subnet 255.255.255.0

- Verify Connectivity

- Open PC0 → Command Prompt

- Type: ping 192.168.1.2

- ✅ Expected Output

- Reply from 192.168.1.2: bytes=32 time<1ms TTL=128

- Success: Indicates both PCs can communicate over the LAN.

- 📘 Learning Outcomes

- Understand basic LAN setup using switch and PCs

- Learn cable types (straight-through)

- Practice IP addressing

- Use ping for testing connectivity